实验七  防火墙IPSec与VPN的配置

姓名：杨佳伲		学号：2022132006		班级：信安实验221班

- 实验目的

- 通过该实验掌握PIX防火墙IPSec与VPN的配置。

- 实验任务

- 配置PIX防火墙IPSec

- 配置PIX防火墙VPN

- 验证实验结果

三、实验设备

- PIX防火墙两台，CISCO 2950交换机两台，控制线一根，网络连接线若干，PC机若干，Sniffer软件一套。

四、实验拓扑图和设备接口

图 1 试验拓扑图

表 1 防火墙 PIX

|  | interface | Type | nameif | Security level | IP Address |
| --- | --- | --- | --- | --- | --- |
| 1 | E0 | physicallogical | inside | 100 | 10.9.9.5 |
| 2 | E1 | physicallogical | outside | 0 | 192.168.9.5 |

表 2 outside-R1-C3700

|  | interface | IP Address |
| --- | --- | --- |
| 1 | F 0/0 | 133.33.33.1 |

表 3 inside-R2-C3700

|  | interface | IP Address |
| --- | --- | --- |
| 1 | F 0/0 | 10.9.9.1 |

表 4 R3-C3700

|  | interface | IP Address |
| --- | --- | --- |
| 1 | F 0/0 | 192.168.3.1 |
| 2 | F 0/1 | 192.168.9.1 |

表 1 路由器 R1/2/3 配置步骤(步骤类似只是ip和route有差别)

| R1 |  |
| --- | --- |
| 步骤 1 | 配置接口 f0/0 R1(config)#int f 0/0 R1(config-if)#ip add 133.33.33.1 255.255.255.0 R1(config-if)#no sh |
| 步骤 2 | 配置路由（R1/2） R1(config)#ip route 0.0.0.0 0.0.0.0 133.33.33.3 R1(config)#ip route 0.0.0.0 0.0.0.0 10.9.9.5 R1(config)#end R1#wr |
| 步骤 3 | 配置telnet连接无密码（R1/2/3） R1(config)#line vty 0 4 R1(config-line)#no login |

表 2 防火墙 pix2 配置步骤

| Pix2 |  |
| --- | --- |
| 步骤 1 | pixfirewall> en Password: pixfirewall# conf t pixfirewall(config)# int e0 pixfirewall(config-if)# ip add 10.9.9.5 255.255.255.0 pixfirewall(config-if)# no sh pixfirewall(config-if)# nameif in pixfirewall(config-if)# nameif ins pixfirewall(config-if)# nameif inside INFO: Security level for "inside" set to 100 by default. pixfirewall(config-if)# no sh pixfirewall(config-if)# exit pixfirewall(config)# int e1 pixfirewall(config-if)# ip add 192.168.9.5 pixfirewall(config-if)# ip add 192.168.9.5 255.255.255.0 pixfirewall(config-if)# nam pixfirewall(config-if)# nameif outside INFO: Security level for "outside" set to 0 by default. pixfirewall(config-if)# no sh pixfirewall(config-if)# end pixfirewall# show ip System IP Addresses: Interface Name IP address Subnet mask Method Ethernet0 inside 10.9.9.5 255.255.255.0 manual Ethernet1 outside 192.168.9.5 255.255.255.0 manual Current IP Addresses: Interface Name IP address Subnet mask Method Ethernet0 inside 10.9.9.5 255.255.255.0 manual Ethernet1 outside 192.168.9.5 255.255.255.0 manual pixfirewall# show int ip br Interface IP-Address OK? Method Status Protocol Ethernet0 10.9.9.5 YES manual up up Ethernet1 192.168.9.5 YES manual up up Ethernet2 unassigned YES unset administratively down up Ethernet3 unassigned YES unset administratively down up Ethernet4 unassigned YES unset administratively down up pixfirewall# wr pixfirewall# conf t pixfirewall(config)# route outside 0.0.0.0 0.0.0.0 192.168.9.1 pixfirewall(config)# |
| 步骤 2 | pixfirewall# conf t pixfirewall(config)# access-list 101 permit ip 10.9.9.0 255.255.255.0 133.33.33.0 255.255.255.0 pixfirewall(config)# access-list aclvpn permit tcp host 133.33.33.1 host 10.9.9.1 pixfirewall(config)# access-list aclvpn permit icmp any any pixfirewall(config)# access-group aclvpn in interface outside pixfirewall(config)# nat (inside) 0 access-list 101 pixfirewall(config)# isakmp policy 10 The DES algorithm requires a VPN-DES activation key. pixfirewall(config)# activation-key 0x6f91ddb5 0x3c8ce153 0xcb588bee 0x234c4f77 |
| 步骤 3 | pixfirewall# conf t pixfirewall(config)# isakmp policy 10 pixfirewall(config-isakmp-policy)# group 1 pixfirewall(config-isakmp-policy)# lifetime 86400 pixfirewall(config-isakmp-policy)# encryption des pixfirewall(config-isakmp-policy)# authentication pre-share pixfirewall(config-isakmp-policy)# hash sha pixfirewall(config-isakmp-policy)# exit pixfirewall(config)# isakmp enable outside pixfirewall(config)# isakmp key cuit123 address 192.168.3.3 pixfirewall(config)# crypto map fw1 10 match address 101 pixfirewall(config)# crypto map fw1 10 set peer 192.168.3.3 pixfirewall(config)# crypto ipsec transform-set fw5 esp-3des pixfirewall(config)# crypto map fw1 10 set transform-set fw5 pixfirewall(config)# crypto map fw1 10 ipsec-isakmp pixfirewall(config)# crypto map fw1 interface outside pixfirewall(config)# no sysopt connection permit-vpn pixfirewall(config)# show run crypto |

表 3 防火墙pix1配置步骤

| PIX1 |  |
| --- | --- |
| 步骤 1 | pixfirewall> en Password: pixfirewall# conf t pixfirewall(config)# int e0 pixfirewall(config-if)# ip add 133.33.33.3 255.255.255.0 pixfirewall(config-if)# no sh pixfirewall(config-if)# nameif inside INFO: Security level for "inside" set to 100 by default. pixfirewall(config-if)# exit pixfirewall(config)# int e1 pixfirewall(config-if)# ip add 192.168.3.3 255.255.255.0 pixfirewall(config-if)# no sh pixfirewall(config-if)# nameif outside INFO: Security level for "outside" set to 0 by default. pixfirewall(config-if)# end pixfirewall# show ip System IP Addresses: Interface Name IP address Subnet mask Method Ethernet0 inside 133.33.33.3 255.255.255.0 manual Ethernet1 outside 192.168.3.3 255.255.255.0 manual Current IP Addresses: Interface Name IP address Subnet mask Method Ethernet0 inside 133.33.33.3 255.255.255.0 manual Ethernet1 outside 192.168.3.3 255.255.255.0 manual pixfirewall# wr pixfirewall# conf t pixfirewall(config)# route outside 0.0.0.0 0.0.0.0 192.168.3.1 |
| 步骤 2 | pixfirewall# conf t pixfirewall(config)# access-list 101 permit ip 133.33.33.0 255.255.255.0 10.9$ pixfirewall(config)# access-list aclvpn permit tcp host 10.9.9.1 host 133.33.$ pixfirewall(config)# access-list aclvpn permit icmp any any pixfirewall(config)# access-group aclvpn in interface outside pixfirewall(config)# nat (inside) 0 access-list 101 pixfirewall(config)# isakmp policy 10 The DES algorithm requires a VPN-DES activation key. pixfirewall(config)# activation-key 0x6f91ddb5 0x3c8ce153 0xcb588bee 0x234c4f77 |
| 步骤 3 | pixfirewall# conf t pixfirewall(config)# isakmp policy 10 pixfirewall(config-isakmp-policy)# group 1 pixfirewall(config-isakmp-policy)# lifetime 86400 pixfirewall(config-isakmp-policy)# encryption des pixfirewall(config-isakmp-policy)# authentication pre-share pixfirewall(config-isakmp-policy)# hash sha pixfirewall(config-isakmp-policy)# exit pixfirewall(config)# isakmp enable outside |
|  | pixfirewall(config)# isakmp key cuit123 address 192.168.9.5 pixfirewall(config)# show run isakmp crypto isakmp enable outside crypto isakmp policy 10 authentication pre-share encryption des hash sha group 1 lifetime 86400 no crypto isakmp nat-traversal pixfirewall(config)# crypto map fw1 10 match address 101 pixfirewall(config)# crypto map fw1 10 set peer 192.168.9.5 pixfirewall(config)# crypto ipsec transform-set fw5 esp-3des pixfirewall(config)# crypto map fw1 10 set transform-set fw5 pixfirewall(config)# crypto map fw1 10 ipsec-isakmp pixfirewall(config)# crypto map fw1 interface outside pixfirewall(config)# no sysopt connection permit-vpn pixfirewall(config)# show run crypto crypto ipsec transform-set fw5 esp-3des esp-none crypto map fw1 10 match address 101 crypto map fw1 10 set peer 192.168.9.5 crypto map fw1 10 set transform-set fw5 crypto map fw1 interface outside crypto isakmp enable outside crypto isakmp policy 10 authentication pre-share encryption des hash sha group 1 lifetime 86400 no crypto isakmp nat-traversal pixfirewall(config)# show run access-group access-group aclvpn in interface outside pixfirewall(config)# show access-list access-list cached ACL log flows: total 0, denied 0 (deny-flow-max 4096) alert-interval 300 access-list 101; 1 elements access-list 101 line 1 extended permit ip 133.33.33.0 255.255.255.0 10.9.9.0 255.255.255.0 (hitcnt=0) 0x8bb037 30 access-list aclvpn; 2 elements access-list aclvpn line 1 extended permit tcp host 10.9.9.1 host 133.33.33.1 eq telnet (hitcnt=0) 0x41d3b41f access-list aclvpn line 2 extended permit icmp any any (hitcnt=0) 0x6dbf3124 pixfirewall(config)# show crypto ipsec sa |

在防火墙上的VPN配置：

（1）配置IKE（ISAKMP）策略

（2）IKE phase1：定义IKE Phase One中的一些策略

（3）定义认证标识（Pre-Shared）

定义认证密码（或者证书）

（4）IKE phase 2：配置IPsec transform

（5）定义感兴趣流量

（6）创建crypto map

（7）将crypto map应用于接口

Show命令验证VPN

注：在配Ipsec的策略时无法配置，原因是show version 后发现VPN-DES等项为Disable，所以无法配置，解决办法是在在防火墙上运行activeation-key命令。

- 实验步骤

5.1：我的实验拓扑图

5.2：R1 的配置 --- IP，静态路由：

5.3：R2 的配置 --- IP，静态路由：

5.4：PIX1/2虚拟防火墙 – IP,安全级别，静态路由

5.5：配置ACL对ipsec数据进行封装

- 实验结果

6.1 【 R1pingR2，R1telnetR2】

6.2 【R2pingR1，R2telnetR1】

七、实验总结

本次实验主要围绕PIX防火墙的IPSec和VPN配置展开，通过实际操作掌握了网络安全设备的基本配置方法，并验证了配置结果的正确性.我深入理解了IPSec VPN的工作原理和配置流程，掌握了PIX防火墙的基本操作命令。实验过程中遇到的配置问题也锻炼了我的故障排查能力。