实验四  防火墙AAA服务的配置

姓名：杨佳伲		学号：2022132006		班级：信安实验221班

- 实验目的

- 通过该实验掌握PIX防火墙AAA服务的配置，熟悉AAA服务器的认证、授权、审计配置。

- 实验任务

- 配置AAA服务器

- 在PIX防火墙上配置AAA服务

- 查看AAA配置，验证配置是否正确

三、实验设备

- 防火墙一台，CISCO 交换机若干，CISCO 路由器若干，控制线一根，网络连接线若干，PC机若干，AAA服务软件一套。

- 实验拓扑图及内容

图 1 试验拓扑图参考

（注：此处aaa服务器选的VMNET8网卡）

表 1 防火墙 PIX

|  | interface | Type | nameif | Security level | IP Address |
| --- | --- | --- | --- | --- | --- |
| 1 | E0 | physicallogical | outside | 0 | 192.168.9.3 |
| 2 | E1 | physicallogical | inside | 100 | 133.33.33.3 |
| 3 | E2 | physicallogical | dmz | 50 | 10.9.9.3 |

表 2 outside-R1-C3700

|  | interface | IP Address |
| --- | --- | --- |
| 1 | F0/0 | 192.168.9.1 |

表 3 inside-PC1-C3700

|  | interface | IP Address |
| --- | --- | --- |
| 1 | F0/0 | 133.33.33.10 |

表 4 DMZ-R2-C3700

|  | interface | IP Address |
| --- | --- | --- |
| 1 | F0/0 | 10.9.9.1 |

表 5 DMZ-R2-C3700

|  | interface | IP Address |
| --- | --- | --- |
| 1 | / | 133.33.33.5 |

表 1 路由器 R1 配置步骤

| R1 |  |
| --- | --- |
| 步骤 1 | 配置接口 f0/0 R1#conf t Enter configuration commands, one per line. End with CNTL/Z. R1(config)#int f 0/0 R1(config-if)#ip add 192.168.9.1 255.255.255.0 R1(config-if)#no sh R1(config-if)#exit |
| 步骤 2 | 配置默认路由 R1(config)#ip route 0.0.0.0 0.0.0.0 192.168.9.3 R1(config)#end R1#wr R1#show ip int br |
| 步骤 3 | 配置密码 R1#conf t R1(config)#line vty 0 4 R1(config-line)#password cisco R1(config-line)#enable password cisco R1(config-line)#end R1#wr |

表 2 路由器 R2 配置步骤

| R2 |  |
| --- | --- |
| 步骤 1 | 配置接口 f0/0 R2#conf t Enter configuration commands, one per line. End with CNTL/Z. R2(config)#int f 0/0 R2(config-if)#ip add 10.9.9.1 255.255.255.0 R2(config-if)#no sh R2(config-if)#exit |
| 步骤 2 | 配置默认路由 R2(config)#ip route 0.0.0.0 0.0.0.0 10.9.9.3 R2(config)#end R2#wr R2#show ip int br |
| 步骤 3 | 配置密码 R2#conf t R2(config)#line vty 0 4 R2(config-line)#password cisco R2(config-line)#enable password cisco R2(config-line)#end R2#wr |

表 3 防火墙配置步骤

| PIX1 |  |
| --- | --- |
| 步骤 1 | 配置三接口ip pixfirewall> en Password: pixfirewall# conf t pixfirewall(config)# host PIX PIX(config)# int e0 PIX(config-if)# nameif inside INFO: Security level for "inside" set to 100 by default. PIX(config-if)# nameif outside PIX(config-if)# ip add 192.168.9.3 255.255.255.0 PIX(config-if)# no sh PIX(config-if)# int e1 PIX(config-if)# ip add 10.9.9.3 255.255.255.0 PIX(config-if)# ip add 133.33.33.3 255.255.255.0 PIX(config-if)# no sh PIX(config-if)# nameif inside INFO: Security level for "inside" set to 100 by default. PIX(config-if)# int e2 PIX(config-if)# ip add 10.9.9.3 255.255.255.0 PIX(config-if)# nameif dmz INFO: Security level for "dmz" set to 0 by default. PIX(config-if)# security-le PIX(config-if)# security-level 50 PIX(config-if)# no sh PIX(config-if)# exit PIX(config)# show int ip br |
| 步骤 2 | 配置(inside,outside) (dmz,outside)静态映射 PIX# conf t PIX(config)# static (inside,outside) 192.168.9.15 133.33.33.5 netmask 255.255.255.255 PIX(config)# static (dmz,outside) 192.168.9.17 10.9.9.1 netmask 255.255.255.255 PIX(config)# end PIX# wr |
| 步骤 3 | 配置ACL列表 PIX(config)# access-list 101 extended permit tcp any any eq www PIX(config)# access-list 101 extended permit tcp any any eq ftp PIX(config)# access-list 101 extended permit tcp any any eq telnet PIX(config)# access-list accacl extended permit tcp any any 添加到组 PIX(config)# access-group 101 in interface outside PIX(config)# access-group 101 in interface dmz PIX(config)#access-list 102 extended permit tcp host 133.33.33.10 host 192.168.9.1 eq telnet 添加aaa认证服务器 PIX(config)# aaa-server aaa1 protocol tacacs+ PIX(config-aaa-server-group)# aaa-server aaa1 host 133.33.33.5 Aaa认证 PIX(config-aaa-server-host)# aaa authentication match 101 outside aaa1 PIX(config)# aaa authentication match 101 dmz aaa1 PIX(config)# aaa authorization match 101 outside aaa1 PIX(config)# aaa authorization match 101 dmz aaa1 PIX(config)# aaa accounting match accacl outside aaa1 PIX(config)# aaa accounting match accacl dmz aaa1 PIX(config)# aaa authentication match 102 inside aaa1 PIX(config)# logging console debugging |
|  | PIX(config)# aaa-server aaa1 host 133.33.33.5 PIX(config-aaa-server-host)# key cisco123 PIX(config-aaa-server-host)# exit PIX(config)# test aaa-server authentication ? PIX(config)# test aaa-server authentication aaa1 Server IP Address or name: 133.33.33.5 Username: user Password: ******** INFO: Attempting Authentication test to IP address <133.33.33.5> (timeout: 12 seconds) INFO: Authentication Successful |

- 实验步骤

- 5.1 【配置AAA服务器】

一、安装 Windows Server 2003

二、安裝/配置 Cicso ACS 4.0

1) 下载并安裝 CiscoSecure ACS 4.0

2) 配置 AAA Client

本地登陆 http://127.0.0.1:2002

进入 AAA Client 管理界面

Add Entry 按钮添加 AAA Clients

Submit + Apply 提交并应用

添加成功

3)配置用户

进入用戶管理

输入  User 点击 Add/Edit 按钮

輸入  Real Name 和 Password （user)  Submit

5.2 【PIX防火墙上配置AAA服务】

具体配置命令如下表：

表 3 防火墙配置步骤

| PIX1 |  |
| --- | --- |
| 步骤 1 | 配置三接口ip pixfirewall> en Password: pixfirewall# conf t pixfirewall(config)# host PIX PIX(config)# int e0 PIX(config-if)# nameif inside INFO: Security level for "inside" set to 100 by default. PIX(config-if)# nameif outside PIX(config-if)# ip add 192.168.9.3 255.255.255.0 PIX(config-if)# no sh PIX(config-if)# int e1 PIX(config-if)# ip add 10.9.9.3 255.255.255.0 PIX(config-if)# ip add 133.33.33.3 255.255.255.0 PIX(config-if)# no sh PIX(config-if)# nameif inside INFO: Security level for "inside" set to 100 by default. PIX(config-if)# int e2 PIX(config-if)# ip add 10.9.9.3 255.255.255.0 PIX(config-if)# nameif dmz INFO: Security level for "dmz" set to 0 by default. PIX(config-if)# security-le PIX(config-if)# security-level 50 PIX(config-if)# no sh PIX(config-if)# exit PIX(config)# show int ip br |
| 步骤 2 | 配置(inside,outside) (dmz,outside)静态映射 PIX# conf t PIX(config)# static (inside,outside) 192.168.9.15 133.33.33.5 netmask 255.255.255.255 PIX(config)# static (dmz,outside) 192.168.9.17 10.9.9.1 netmask 255.255.255.255 PIX(config)# end PIX# wr |
| 步骤 3 | 配置ACL列表 PIX(config)# access-list 101 extended permit tcp any any eq www PIX(config)# access-list 101 extended permit tcp any any eq ftp PIX(config)# access-list 101 extended permit tcp any any eq telnet PIX(config)# access-list accacl extended permit tcp any any 添加到组 PIX(config)# access-group 101 in interface outside PIX(config)# access-group 101 in interface dmz PIX(config)#access-list 102 extended permit tcp host 133.33.33.10 host 192.168.9.1 eq telnet 添加aaa认证服务器 PIX(config)# aaa-server aaa1 protocol tacacs+ PIX(config-aaa-server-group)# aaa-server aaa1 host 133.33.33.5 Aaa认证 PIX(config-aaa-server-host)# aaa authentication match 101 outside aaa1 PIX(config)# aaa authentication match 101 dmz aaa1 PIX(config)# aaa authorization match 101 outside aaa1 PIX(config)# aaa authorization match 101 dmz aaa1 PIX(config)# aaa accounting match accacl outside aaa1 PIX(config)# aaa accounting match accacl dmz aaa1 PIX(config)# aaa authentication match 102 inside aaa1 PIX(config)# logging console debugging |
|  | PIX(config)# aaa-server aaa1 host 133.33.33.5 PIX(config-aaa-server-host)# key cisco123 PIX(config-aaa-server-host)# exit PIX(config)# test aaa-server authentication ? PIX(config)# test aaa-server authentication aaa1 Server IP Address or name: 133.33.33.5 Username: user Password: ******** INFO: Attempting Authentication test to IP address <133.33.33.5> (timeout: 12 seconds) INFO: Authentication Successful |

截图配置情况：

1）三接口配置

- acl配置

5.3 【看AAA配置，验证配置是否正确】

查看防火墙配置信息

- 实验结果

6.1 【 对应试验结果1】

6.2 【对应试验结果2】

6.3 【对应试验结果3】

防火墙认证成功

R1telnet连接R2用户user成功认证和授权

R2telnet连接R1用户user成功认证和授权

七、实验总结(Q&A)

Summary：本人通过多次查找资料排查错误，完成了实验，成功配置PIX防火墙的AAA认证服务。PIX防火墙AAA服务的配置，熟悉AAA服务器的认证、授权、审计配置。通过ACS模拟认证服务器的使用，我们可以看到防火墙可以实现认证服务，对过往流量监控和用户进行认证,实验中使用了下列参考资料。

[1]思科交换机 ACS4.0 Tacacs+ 登陆验证_cisco secure acs4可以安装到win吗-CSDN博客